AHD  REAR OBSERVATION/ BACKUP

# CAMERA SYSTEM

OPERATION AND  INSTALLATION  MANUAL

# INTRODUCTION

Thank you for purchasing our product ,  please  read  it thoroughly  before operating and  keep  it for future  reference .

The AHD  Rear Vision System will  improve your ability to see  behind your car , pickup truck , camper , travel trailer , fifth wheel trailer or motorhome . We  have taken  numerous  measures

to ensure that your product  is delivered  in top condition and will  perform to your satisfaction .

IMPORTANT SAFETY INSTRUCTIONS

Before you install

If you are  not confident working with  12V/24V  DC vehicle wiring , contact your vehicle manufacturer or your nearest  retailer/installer to get a  professional  installation .

Repair

The camera system should  not  be opened. Any attempt at  modification or repair by the user will void the warranty.

PARTS

1. Monitor                                                                                   2. Camera

3.49 .2 ft Video Cable

cigarette lighter

Suction cup mount

User Manual

4.Accessories

Camera bracket

| Screws & Bolt |  |
| --- | --- |

Power Cord

2 In 1 Cigarette Lighter Socket

# Attention: The above accessories list may have little differences for different retail platforms ,

we may increase or update some accessories without notifying in advance , please subject to the actual received package .

These  instructions do  not apply to all vehicles . They are only  meant as a general

guide due to the  number of different  makes and  models . For vehicle specific questions , contact your vehicle  manufacturer.

Installation

Camera Installation Caution

Make sure there are  no electrical cables , gas  lines or  important  parts  behind where the drill  holes will  be .  Make sure to  isolate the  12V or 24V  power source  by disconnecting   the  negative  (-) terminal  from the  battery.

Metal  brackets for plain surfaces

screw TA4x8

Allen Screw 3x6

Washer ￠4

Camera Positioning

Camera  location

Wiring Installation

The system can  be  used as a  rear observation system or as a  backup camera system .

·To use it as a rear observation system , you can connect the camera with either CH1 or CH2 and no need to connect the trigger wire; To use the system as a backup camera system , the camera

needs to be connected with CH2 and the trigger wire should be connected with the reversing light's 12V.

# 1 .Choose  routing  path for the camera 's  power cable to the  power source depending on the desired

function .

2 .Before drilling the  hole ,make sure there are  no components  behind the surface you are drilling such as electrical cables ,gas  lines ,or other important components that can  be damaged.

3 .After drilling the  hole ,insert the supplied grommet then  route the camera cable  into the opening. The grommet  must  be  used to  prevent the  metal edge from cutting the cable .

4 .Locate the  power source  required :Backup  lights  if used as a  backup camera or running/marker lights  is used as a  rear/front observation system .

5.Once you  have  located the  power source ,route the camera cable to that location . Never  route  the cable on the outside of the vehicle .

6 .Connect wires as diagram  below.

Monitor Installation

Make sure the  monitor is  mounted  in a  location that will  not obstruct your vision while driving.

1.On-dash  Mounting                                   2.Suction cup Mounting

Attention: The trigger signal wire require professional techinical person to install , if you use it as rear observation system , please ignore it.

Reversing Trigger Wire

cigarette lighter

When the green wire is activated , the monitor automatically switches to Video 2 .

# OPERATION

1. Power Supply

Power to the  monitor is supplied with the vehicle's  12V/24V  power socket.

2. Monitor Operation

The  buttons  include -,Menu ,+ ,as shown  below :

+  	 MENU

| + | Select between AV1&AV2 Increase the settings value . |
| --- | --- |
| MENU | Short press to enter the OSD menu , long press to turn on or off the display. |
| - | Decrease the settings value . |

# Press the MENU button to enter in main menu ,Short press the MENU button to select

Brightness / Contrast / Color / Languge / Default Setting/ RLUD/ Ruler Setting /Version

1. BRIGHTNESS

Press the “ + ” or “ - ” button to adjust the screen brightness .

2.CONTRAST

Press the " +" or "-" button to adjust the contrast.

3.COLOR

Press the " +" or "-" button to adjust the color

4. LANGUAGE

English only

5.DEFAULT SETTING

# Press the " +" or "-" button to adjust the Default Setting

6. UN-MIRRORED IMAGE

Press the " +" or "-" button to set the mirror (backup camera) or un-mirrored image (front camera)

6.RULER  SETTING

Press the " +" or "-" button to SET the Rear Assit Marker Lines on or off on CH1 or CH2 .

4.VERSION

TECHNICAL  SPECIFICATIONS

| Camera |  |
| --- | --- |
| Image Sensor | 1/3 Color CMOS |
| # Waterproof Rate | IP69K |
| Power Supply | +12V |
| Current Consumption | TYPE:170mA , Max .250mA |
| Resolution | 2 .0 Mega 1080P |
| Frame Rate | 25f/s 30f/s |
| Operating Temperature | -30~ +70 ºC |
| Humidity | 20% ~ 80% |
| LCD monitor |  |
| LCD display screen size | 5 inch ( IPS Panel) |
| Resolution | 800*480 |
| Power Supply | +12~ +24V |
| Output Voltage | 12V |
| Current Consumption | TYPE:380mA , Max .480mA |
| Video input | 720P/ 1080P |
| Power consumption | Normal working status <5W |
| Operating Temperature | -30~ +70 ºC |
| Humidity | 20% ~ 80% |

Waste electrical  products should  not  be disposed of with  household waste .

Please  recycle where facilities exist. Check with

your local authority or retailer for recycling advice .